HDR / SDR Calibration Slot Persistence

VPL-VW385ES  ·  How Gamma Slots and Settings Are Stored by Signal Type  ·  Verified from Operating Instructions (47254410M)

1  The Core Question

When you upload a custom gamma curve (.ldt file) to one of the Gamma 7–10 slots via Sony ImageDirector, does that curve survive when you switch between an SDR signal (Rec.709) and an HDR signal (Rec.2020 / HDR10)? And does the projector maintain separate settings for each signal type?

KEY FINDING   The uploaded curve in any Gamma slot (7–10) is stored permanently in that slot. Switching signal types does NOT erase or overwrite it. The slot holds the curve until you deliberately overwrite it with another upload.

2  How the Projector Organizes Its Memory

The projector uses a two-level storage system. Understanding this is essential for planning where to save calibration settings for each signal type.

2.1  Level 1 — Preset Memory Numbers (Signal-Driven)

The projector automatically assigns an internal preset memory number to every incoming signal based on its resolution and frame rate. This happens automatically — you cannot choose or name these numbers. They are the projector's way of recognizing signal types.

| PRESET NUMBERS | RESOLUTION / SIGNAL | TYPICAL USE |
| --- | --- | --- |
| 5–14 | 480p, 576p, 720p, 1080i, 1080p (HD) | SDR Blu-ray, streaming SDR, broadcast |
| 74–79 | 3840×2160 & 4096×2160 (4K UHD, 50/60Hz) | HDR10 streaming, 4K sources |
| 93–96 | 3840×2160 & 4096×2160 (4K UHD, 24/25/30Hz) | HDR10 Blu-ray, cinema-rate 4K |
| 18–20, 71–72 | 3D signals | Active 3D content |
| 26–55 | Computer/PC signals | PC input |

PRACTICAL MEANING   Your SDR 1080p Blu-ray player triggers preset numbers 5–14. Your HDR10 4K source triggers preset numbers 74–79 or 93–96. These are different memory banks — they do not share settings.

2.2  Level 2 — Calib. Preset (User-Driven)

Within each preset memory number, the projector stores settings per Calib. Preset mode. The manual confirms: Gamma Correction, Color Space, Color Temp, CMS, and HDR setting are all stored "for each input connector and each Calib. Preset." This gives you 9 independent setting profiles per input.

| WHAT IS STORED PER | STORAGE SCOPE |
| --- | --- |
| Gamma Correction (slot selection) | Per input connector + per Calib. Preset |
| Color Space | Per input connector + per Calib. Preset |
| Color Temp (Gain/Bias R/G/B) | Per Color Temp slot (D93–D55, Custom 1–5) |
| Color Correction CMS | Per input connector + per Calib. Preset |
| HDR setting | Per input connector + per Calib. Preset |
| Calib. Preset selection | Per input connector |
| Gamma slot curve data (7–10) | Global — one copy, shared across all presets |

CRITICAL DISTINCTION   The gamma curve data stored in each slot (7–10) is GLOBAL — it is shared across all Calib. Presets and all inputs. Only the SELECTION of which slot to use is stored per preset. This means: if you overwrite Gamma 7 with a new curve, every preset that was using Gamma 7 will now use the new curve.

3  Memory Map — What Lives Where

GAMMA SLOT DATA (Global — permanent until overwritten)

┌──────────────┬──────────────┬──────────────┬──────────────┐

│  Gamma 7     │  Gamma 8     │  Gamma 9     │  Gamma 10    │

│  (your curve)│  (your curve)│  (your curve)│  (spare)     │

└──────────────┴──────────────┴──────────────┴──────────────┘

HDMI 1 INPUT — HD signal (preset 5–14) = SDR source

├─ Calib. Preset: Reference → Gamma: Slot 7, CS: BT.709, HDR: Off

├─ Calib. Preset: Cinema Film 2 → Gamma: Slot 9, CS: BT.2020, HDR: Off

└─ (other presets independent)

HDMI 1 INPUT — 4K signal (preset 74–96) = HDR source

├─ Calib. Preset: Cinema Film 1 → Gamma: Slot 8, CS: BT.2020, HDR: HDR10

└─ (other presets independent)

4  What Happens When You Switch Signal Types

4.1  Different Sources on Different HDMI Ports

This is the ideal setup. Each port maintains its own independent settings.

| HDMI PORT | SOURCE | PRESET NUMBERS | RECALLS |
| --- | --- | --- | --- |
| HDMI 1 | 4K HDR media player | 74–96 | Cinema Film 1 → Gamma 8, BT.2020, HDR10 |
| HDMI 2 | SDR 1080p Blu-ray | 5–14 | Reference → Gamma 7, BT.709, HDR Off |

BEST PRACTICE   Switching inputs automatically loads the correct Calib. Preset settings for that source. No manual switching needed — the ideal configuration.

4.2  SDR and HDR from the Same Device on One Port

If a single device (e.g. Apple TV 4K, media player) outputs both SDR and HDR content on the same HDMI port, the projector may still detect different preset memory numbers if the resolution changes (e.g. SDR at 1080p vs HDR at 4K). However, if both are 4K resolution, they share the same preset memory number.

| SCENARIO | PRESET NUMBER | SETTING RECALL | ACTION REQUIRED |
| --- | --- | --- | --- |
| SDR source outputs 1080p, HDR outputs 4K | Different (5–14 vs 74–96) | Different Calib. Presets load automatically | None — works automatically |
| Both SDR and HDR output at 4K from same device | Same (74–96) | Same Calib. Preset is recalled for both | Manually switch Calib. Preset when changing content type |

WARNING — HDR AUTO   When HDR Auto is set to On, the projector reads HDR metadata from the signal and can override your Color Space setting (forces BT.2020 for HDR10 signals). This makes calibration unpredictable. Set HDR manually in each calibrated Calib. Preset.

5  Quick Reference Summary

| QUESTION | ANSWER |
| --- | --- |
| Does the uploaded curve survive a signal change? | Yes — Gamma slot data is global and permanent until overwritten |
| Are there separate HDR and SDR gamma slots? | No — the 4 slots (7–10) are shared. You assign which slot each Calib. Preset uses |
| Are settings stored separately for HD vs 4K signals? | Yes — different preset memory numbers (5–14 for HD, 74–96 for 4K) each have independent Calib. Preset storage |
| Does switching Color Space to BT.2020 erase Gamma slot data? | No — Color Space and Gamma Correction are independent settings |
| If I overwrite Gamma 7 with a new curve, does it affect other presets? | Yes — slot data is global. Any preset pointing to Gamma 7 will use the new curve |

Sony VPL-VW385ES  ·  HDR/SDR Calibration Slot Persistence  ·  Verified from Operating Instructions (47254410M)