VPL-VW385ES — Three Calibration Scenarios

Complete Planning Guide  ·  SDR Rec.709  ·  HDR Rec.2020 PQ  ·  SDR DCI-P3  ·  Sony VPL-VW385ES

Overview

This guide covers three distinct calibration targets for the Sony VPL-VW385ES. Each scenario requires a different combination of Color Space, Gamma Correction slot, HDR setting, and Calib. Preset. The goal is to store each calibration independently so they can be recalled without interference.

| SCENARIO 1 SDR · Rec.709 Gamma 2.4 / BT.1886 | SCENARIO 2 HDR · Rec.2020 PQ Curve | SCENARIO 3 SDR · DCI-P3 Gamma 2.4 / BT.1886 |
| --- | --- | --- |
| Calib. Preset: Reference Gamma Slot: 7 Color Space: BT.709 HDR: Off | Calib. Preset: Cinema Film 1 Gamma Slot: 8 Color Space: BT.2020 HDR: HDR10 | Calib. Preset: Cinema Film 2 Gamma Slot: 9 Color Space: BT.2020 + CMS HDR: Off |

1  Color Space Menu — What Exists on This Projector

Before planning the scenarios, it's important to know exactly what Color Space options the projector provides — and what is notably absent.

| MENU OPTION | TYPE | GAMUT COVERAGE | USE FOR |
| --- | --- | --- | --- |
| BT.709 | Fixed standard | Rec.709 / sRGB | Scenario 1 — SDR broadcast and Blu-ray |
| BT.2020 | Fixed standard | Full UHD wide gamut | Scenario 2 & 3 — HDR and P3 (starting point) |
| Color Space 1 | Sony preset | TV/sports tuning | General TV use — not for calibration |
| Color Space 2 | Sony preset | Bright room TV tuning | General TV use — not for calibration |
| Color Space 3 | Sony preset | Bright room cinema | General use — not for calibration |
| Custom | User-adjustable | Global shift only (2 sliders) | Minor gamut trim only — not P3 capable |
| DCI-P3 | ⚠ NOT AVAILABLE | — | Must be achieved via BT.2020 + CMS corrections |

IMPORTANT   There is no DCI-P3 color space preset on the VW385ES. P3 calibration requires selecting BT.2020 as the color space and using the Color Correction CMS to pull each primary and secondary to P3 target coordinates.

2  Gamma Slot Assignment

The four custom gamma slots (7–10) hold uploaded .ldt curves. Assign one slot per scenario to keep them fully independent. The built-in gamma options (1.8 / 2.0 / 2.1 / 2.2 / 2.4 / 2.6) do not occupy these slots.

| SLOT | ASSIGNED SCENARIO | CURVE TYPE | GENERATED WITH |
| --- | --- | --- | --- |
| Gamma 7 | Scenario 1 — SDR Rec.709 | BT.1886, Lb ≈ 0.5% | LDT Editor → BT.1886 preset, Lb slider at 0.5% |
| Gamma 8 | Scenario 2 — HDR PQ | PQ tone map, ~200 nit | LDT Editor → PQ preset, nit slider at 200 |
| Gamma 9 | Scenario 3 — SDR DCI-P3 | BT.1886, Lb ≈ 0.5% | Same curve as Gamma 7 — separate slot for independence |
| Gamma 10 | (Spare) | — | Reserve for future use or experimentation |

WHY TWO SLOTS FOR THE SAME CURVE   Scenarios 1 and 3 use the same BT.1886 curve but stored in separate slots (7 and 9). This ensures that adjusting or re-uploading the curve for one scenario never accidentally affects the other.

SCENARIO 1  —  SDR · Rec.709 · Gamma 2.4 (BT.1886)

| SETTING | VALUE | REASON |
| --- | --- | --- |
| Calib. Preset | Reference | Designed for faithful, unprocessed source reproduction — ideal for SDR calibration baseline |
| HDR | Off (manual) | Hard-set Off — prevents Auto mode from overriding Color Space when a BT.2020-flagged signal appears |
| Color Space | BT.709 | Correct gamut container for Rec.709 SDR content |
| Gamma Correction | Gamma 7 | BT.1886 curve uploaded via LDT Editor. More accurate than built-in 2.4 — compensates for projector's non-zero black level |
| Color Temp | Custom 3 (D65) | D65 is the Rec.709 white point. Custom 3 factory default matches D65 |
| Color Correction | CMS to Rec.709 targets | After setting Gamma and Color Temp, measure primaries/secondaries and dial in CMS corrections |
| Lamp Control | Low (recommended) | Low lamp deepens blacks, improves contrast for dark-room SDR viewing |

BT.1886 vs BUILT-IN 2.4   Why BT.1886 instead of the built-in 2.4? The built-in gamma 2.4 is a pure power law that assumes a perfect black level of zero. The VW385ES has a very low but non-zero black level. BT.1886 accounts for this with a soft toe near black, giving more accurate shadow rendering that matches how reference monitors handle Rec.709 content.

SCENARIO 2  —  HDR · Rec.2020 · PQ Curve

| SETTING | VALUE | REASON |
| --- | --- | --- |
| Calib. Preset | Cinema Film 1 | Cinema-oriented preset, natural home for HDR mastered theatrical content |
| HDR | HDR10 (manual) | Set manually to HDR10 — do NOT use Auto. Auto can override Color Space mid-calibration and causes unpredictable behavior |
| Color Space | BT.2020 | HDR10 content is delivered in the BT.2020 container |
| Gamma Correction | Gamma 8 | PQ tone-mapping curve uploaded via LDT Editor, targeting ~200 nit (VW385ES peak brightness) |
| Color Temp | Custom 3 (D65) | HDR10 mastering white point is D65 |
| Color Correction | CMS to P3-D65 targets | HDR10 content is typically mastered within a P3-D65 gamut inside the BT.2020 container — target P3 primaries, not full BT.2020 |
| Lamp Control | High | HDR requires maximum brightness headroom for specular highlights |

CMS TARGET FOR HDR   Calibrate CMS to P3-D65 primary coordinates, not full BT.2020 primaries. The vast majority of HDR10 content is graded within a P3 gamut and delivered in a BT.2020 container. Calibrating to full BT.2020 primaries will result in oversaturated colors for most real-world HDR content.

PQ Curve Generation (LDT Editor):

1. Open LDT Editor → Sidebar → HDR PQ section

2. Set nit slider to 200 (VW385ES peak brightness)

3. Click "Generate PQ → 200 nit"

4. Export As → name file "HDR_PQ_200nit.ldt"

5. Open Sony ImageDirector → load file → upload to Gamma 8 slot

SCENARIO 3  —  SDR · DCI-P3 · Gamma 2.4 (BT.1886)

| SETTING | VALUE | REASON |
| --- | --- | --- |
| Calib. Preset | Cinema Film 2 | Cinema Film 2 is appropriate for theatrical P3 content — distinct from Reference (Rec.709) |
| HDR | Off (manual) | SDR content — hard-set Off |
| Color Space | BT.2020 | No P3 preset exists. Start from BT.2020 (widest gamut) and trim to P3 targets via CMS. Starting from BT.709 would clip P3 primaries before CMS can correct them |
| Gamma Correction | Gamma 9 | Same BT.1886 curve as Scenario 1 but in a separate slot for independence |
| Color Temp | Custom 3 (D65) | P3-D65 is the standard for home/streaming P3 content. Use D63 only for theatrical DCI alignment |
| Color Correction | CMS to P3-D65 targets | This is how P3 is achieved — measure each primary/secondary and correct to P3 chromaticity coordinates |
| Lamp Control | Low (recommended) | Dark-room SDR viewing |

WHY BT.2020 FOR P3   Why BT.2020 as starting point for P3?
P3 primaries lie OUTSIDE Rec.709 but INSIDE BT.2020. If you start from BT.709, the projector clips any color that exceeds Rec.709 before the CMS can correct it — you can never reach P3. Starting from BT.2020 leaves the full P3 gamut accessible for CMS to shape.

DCI-P3 Primary Targets (D65 white point):

Red:     x=0.680  y=0.320

Green:   x=0.265  y=0.690

Blue:    x=0.150  y=0.060

White:   x=0.3127 y=0.3290  (D65)

Use these as CMS targets when measuring with a colorimeter.

Rec.709 Red (x=0.640) and Green (x=0.300) are notably inside P3 —

those two primaries will require the most CMS correction.

3  Side-by-Side Settings Comparison

| SETTING | SCENARIO 1 SDR REC.709 | SCENARIO 2 HDR REC.2020 | SCENARIO 3 SDR DCI-P3 |
| --- | --- | --- | --- |
| Calib. Preset | Reference | Cinema Film 1 | Cinema Film 2 |
| HDR | Off | HDR10 | Off |
| Color Space | BT.709 | BT.2020 | BT.2020 |
| Gamma Slot | Gamma 7 | Gamma 8 | Gamma 9 |
| Gamma Curve | BT.1886, Lb 0.5% | PQ 200 nit | BT.1886, Lb 0.5% |
| Color Temp | Custom 3 (D65) | Custom 3 (D65) | Custom 3 (D65) |
| CMS Targets | Rec.709 primaries | P3-D65 primaries | P3-D65 primaries |
| Lamp Control | Low | High | Low |

4  Calibration Procedure Order  (Same for All Scenarios)

Always calibrate in this sequence. Each step depends on the previous being locked first.

| 1 | Color Space + Cyan–Red / Magenta–Green | Foundation — sets where the gamut triangle sits. Lock this first. Any change here invalidates all steps below. |
| --- | --- | --- |
| 2 | Color Temp Gain R/G/B + Bias R/G/B | Sets white point and black point within the gamut. Measure with colorimeter and adjust Custom Color Temp sub-controls. |
| 3 | Gamma Correction (upload .ldt curve) | Sets tonal response. Upload appropriate .ldt via ImageDirector. Verify with EOTF measurement. |
| 4 | Color Correction (CMS) Hue / Saturation / Brightness per color | Fine-tune individual primaries and secondaries. Measure each color at 100% saturation and correct to target. |
| 5 | Verify saturation sweep 25% / 50% / 75% / 100% | CMS corrections at 100% do not translate perfectly to lower saturations. Measure all levels and accept residual error within ΔE < 3. |
| 6 | Save Calib. Preset | Navigate to Calib. Preset → confirm all settings are correct → the projector saves automatically. Verify by recalling the preset. |

CALIBRATION ORDER RULE   If you ever go back and adjust the Color Space or Cyan–Red/Magenta–Green slider (Step 1), you must redo Steps 2 through 6 entirely. Steps 3–6 are always downstream of the gamut foundation.

5  Day-to-Day Switching Workflow

If HDR and SDR sources are on different HDMI ports:

Switch HDMI input  →  Projector auto-loads the correct Calib. Preset  →  Done.

No manual action required.

If both come from one device on one HDMI port:

Content changes to HDR  →  Press CALIBRATED PRESET → select Cinema Film 1

Content changes to SDR Rec.709  →  Press CALIBRATED PRESET → select Reference

Content changes to SDR P3  →  Press CALIBRATED PRESET → select Cinema Film 2

The CALIBRATED PRESET button on the remote control makes this a single button press.

Sony VPL-VW385ES  ·  Three Calibration Scenarios Planning Guide  ·  Based on Operating Instructions (47254410M) and OSD verification