VPL-VW385ES  ·  Picture & Color Calibration Controls

Sony 4K SXRD Home Cinema Projector  ·  Verified from Operating Instructions (47254410M)

1  Calibrated Preset  (Picture Modes)

Nine picture modes selectable via the CALIBRATED PRESET buttons on the remote or the Calib. Preset item in the Picture Menu. Each mode stores independent settings per input connector.

| MODE | INTENDED USE |
| --- | --- |
| Cinema Film 1 | Reproduces the dynamic, clear look of master positive film — primary cinema mode |
| Cinema Film 2 | Rich tone and color of a movie theater; based on Cinema Film 1 |
| Reference | Faithful reproduction of source material; no processing applied — ideal for calibration |
| TV | TV programs, sports, concerts, general video content |
| Photo | Still images from a digital camera |
| Game | Optimised for gaming; well-modulated colours and fast response |
| Bright Cinema | Movie watching in a bright room such as a living room |
| Bright TV | TV/sports in a bright room |
| User | Fully user-customisable; factory default matches Reference |

TIP  All picture quality adjustments (Contrast, Brightness, Color, Gamma, etc.) are saved independently for each input connector and each Calib. Preset mode.

2  Picture Menu

Primary adjustment controls accessible from the main Picture menu.

2.1  Image Processing

| CONTROL | OPTIONS | NOTES |
| --- | --- | --- |
| Reality Creation | On / Off | Super-resolution. Sub-controls: Database (Normal / Mastered in 4K), Resolution level, Noise Filtering level |
| Advanced Iris | Full / Limited / Off | Dynamic iris under Cinema Black Pro. Full = maximum dynamic range; Limited = slower, darker, better for very dark rooms |
| Contrast Enhancer | High / Middle / Low / Off | Automatically corrects bright/dark balance per scene to optimise contrast and perceived sharpness |
| Lamp Control | High / Low | High = maximum brightness; Low = deeper blacks, quieter fan, longer lamp life |
| Motionflow | Impulse / Combination / Smooth High / Smooth Low / True Cinema / Off | Frame interpolation engine. True Cinema preserves 24fps cadence exactly |

2.2  Core Tone & Color Controls

| CONTROL | RANGE / OPTIONS | NOTES |
| --- | --- | --- |
| Contrast | Slider (HDR: "Contrast(HDR)") | White-point adjustment. When HDR10/HLG/Auto HDR is active, label changes to Contrast(HDR) |
| Brightness | Slider | Black-lift adjustment. Set this first, then configure Dynamic Control for room brightness |
| Color | Slider | Master saturation — affects all hues equally |
| Hue | Slider (–red / +green) | Master hue rotation |
| Sharpness | Slider | Remote shortcut: SHARPNESS +/–. Higher = sharper edges; Lower = softer, less noise |

2.3  Color Temperature

Four fixed presets plus five fully user-adjustable custom slots. Each slot stores independent Gain and Bias values for all three channels.

| PRESET | COLOUR TEMP. | CHARACTER |
| --- | --- | --- |
| D93 | ~9 300 K | Blue-tinted whites; standard TV colour temperature in Japan |
| D75 | ~7 500 K | Neutral tint between D93 and D65 |
| D65 | ~6 500 K | Standard illuminant for broadcast and cinema mastering |
| D55 | ~5 500 K | Warm, reddish whites |
| Custom 1 | Factory = D93 | User-adjustable |
| Custom 2 | Factory = D75 | User-adjustable |
| Custom 3 | Factory = D65 | User-adjustable |
| Custom 4 | Factory = D55 | User-adjustable |
| Custom 5 | Brightness priority | User-adjustable |

Each of the 9 colour temperature entries (D93–D55 and Custom 1–5) exposes 6 sub-controls:

| SUB-CONTROL | AFFECTS | FUNCTION |
| --- | --- | --- |
| Gain R | White point — Red | Sets how red the peak-white appears |
| Gain G | White point — Green | Sets how green the peak-white appears |
| Gain B | White point — Blue | Sets how blue the peak-white appears |
| Bias R | Black point — Red | Lifts or lowers the red channel at black |
| Bias G | Black point — Green | Lifts or lowers the green channel at black |
| Bias B | Black point — Blue | Lifts or lowers the blue channel at black |

NOTE  6 sub-controls × 9 colour temperature slots = up to 54 individual colour temperature adjustments.

3  Expert Setting  (Picture Menu Sub-Section)

Accessed within the Picture Menu. These controls are hidden at the basic level and require navigating into the Expert Setting sub-menu.

3.1  Noise & Processing

| CONTROL | OPTIONS | NOTES |
| --- | --- | --- |
| NR (Noise Reduction) | Auto / High / Middle / Low / Off | Reduces analogue-style noise and picture roughness |
| MPEG NR | Auto / High / Middle / Low / Off | Targets block noise and mosquito noise from digital compression |
| Smooth Gradation | High / Middle / Low / Off | Smooths banding in flat gradient areas of the image |
| Film Mode | Auto / Off | Detects and handles film-sourced progressive content |

3.2  Gamma Correction

Eleven selectable gamma settings plus Off. The six standard power-law presets cover the broadcast/cinema range; four custom slots (Gamma 7–10) are uploaded via Sony ImageDirector software as .ldt files.

| SETTING | TYPE | CHARACTER |
| --- | --- | --- |
| 1.8 | Power law | Brightest overall — suitable for very bright rooms |
| 2.0 | Power law | Moderately bright |
| 2.1 | Power law | Between 2.0 and 2.2 |
| 2.2 | Power law | Broadcast SDR standard (sRGB / BT.709 display gamma) |
| 2.4 | Power law | BT.1886 ideal — darker mid-tones, suited for dark rooms |
| 2.6 | Power law | Darkest overall — maximum shadow detail visibility |
| Gamma 7 | Custom LDT | Simulates film gamma curve (Sony description). Upload via ImageDirector |
| Gamma 8 | Custom LDT | Increases image sharpness; for bright viewing environments |
| Gamma 9 | Custom LDT | Brighter than Gamma 8; for bright viewing environments |
| Gamma 10 | Custom LDT | Increases sharpness; for watching TV in bright rooms |
| Off | — | No gamma correction applied |

EDITOR NOTE  Gamma 7–10 are the custom slots targeted by this LDT editor. The Sony descriptions above are factory defaults; the slots accept any uploaded curve.

3.3  Color Correction

A per-axis 6-colour correction matrix. For each colour axis, three properties can be adjusted independently while watching the projected image.

| COLOUR AXIS | HUE | SATURATION | BRIGHTNESS |
| --- | --- | --- | --- |
| Red | ✓ | ✓ | ✓ |
| Yellow | ✓ | ✓ | ✓ |
| Green | ✓ | ✓ | ✓ |
| Cyan | ✓ | ✓ | ✓ |
| Blue | ✓ | ✓ | ✓ |
| Magenta | ✓ | ✓ | ✓ |

NOTE  6 colour axes × 3 properties = 18 individual colour correction adjustments.

3.4  Color Space

Selects or customises the colour gamut. When "Custom" is selected, a per-colour chromaticity adjustment sub-menu becomes available.

| MODE | DESCRIPTION |
| --- | --- |
| BT.709 | ITU-R BT.709 — standard for HD broadcast and Blu-ray. Equivalent to sRGB |
| BT.2020 | Wide colour gamut for HDR content. Automatically selected when HDR Auto detects a BT.2020 signal |
| Color Space 1 | Sony-tuned preset for TV programs, sports, and concert video |
| Color Space 2 | Sony-tuned preset for TV/sports in a bright room |
| Color Space 3 | Sony-tuned preset for movies in a bright room |
| Custom | Full user control — exposes the chromaticity adjustment sub-menu below |

Custom Color Space — Chromaticity Sub-Menu

When Custom is selected, a Color Select control cycles through six colour axes. Each axis exposes two sliders that shift the selected primary or secondary colour along opponent-colour directions in the CIE chromaticity diagram.

| COLOR SELECT | SLIDER 1 | SLIDER 2 |
| --- | --- | --- |
| Red | Cyan – Red | Magenta – Green |
| Green | Cyan – Red | Magenta – Green |
| Blue | Cyan – Red | Magenta – Green |
| Cyan | Cyan – Red | Magenta – Green |
| Magenta | Cyan – Red | Magenta – Green |
| Yellow | Cyan – Red | Magenta – Green |

Note: Cyan–Red moves the selected colour toward cyan (negative) or red (positive). Magenta–Green moves it toward magenta (negative) or green (positive). Together these two axes allow 2D chromaticity positioning of each colour.

VERIFIED OSD  6 colour axes × 2 sliders = 12 individual chromaticity adjustments in Custom Color Space. Verified from projector on-screen display (screenshot). The manual only states "You can adjust the color space setting" without listing the sub-controls.

3.5  Remaining Expert Controls

| CONTROL | OPTIONS | NOTES |
| --- | --- | --- |
| Clear White | High / Low / Off | Emphasises vivid whites |
| x.v.Color | On / Off | For x.v.Color-capable sources. Enabling this disables gamma correction |
| HDR | Auto / HDR10 / HLG / Off | Auto detects HDR metadata and sets optimal processing. BT.2020 Color Space is set automatically when Auto detects a BT.2020 signal |
| Input Lag Reduction | On / Off | Reduces display latency. When On, Motionflow, NR, and MPEG NR are disabled |

4  Advanced Picture Menu

Separate menu section for panel-level auto calibration. Compensates for colour drift that occurs after extended use of the SXRD panels.

| FUNCTION | DESCRIPTION |
| --- | --- |
| Pre Check | Measures current colour against factory reference. Reports dE (colour difference) before correction |
| Adjust | Performs automatic calibration. Lens returns to factory default position during process (~few minutes) |
| Before/After | Toggles between factory and post-calibration settings at a fixed frequency so you can evaluate the effect on a live image |
| Reset | Discards calibration results and reverts to factory default panel settings |

Note: Perform calibration after 30+ minutes warm-up. Avoid turning off power or operating controls during Pre Check/Adjust.

5  Total Adjustment Count Summary

Approximate total of distinct calibration-relevant adjustments, verified from the manual.

| CATEGORY | COUNT | NOTES |
| --- | --- | --- |
| Calib. Preset modes | 9 | Cinema Film 1/2, Reference, TV, Photo, Game, Bright Cinema, Bright TV, User |
| Core tone controls (Contrast/Brightness/Color/Hue/Sharpness) | 5 | Basic level |
| Image processing toggles (Reality Creation, Iris, Contrast Enhancer, Lamp, Motionflow) | 5 | Each has multiple sub-options |
| Colour Temperature presets | 4 fixed + 5 custom = 9 | D93, D75, D65, D55, Custom 1–5 |
| Colour Temperature Gain/Bias sub-controls | 6 per slot × 9 slots = 54 | Full expert colour temp control |
| Gamma Correction presets | 10 + Off | 6 standard + 4 custom LDT slots (Gamma 7–10) |
| Colour Correction matrix | 18 | 6 colours × Hue, Saturation, Brightness |
| Noise & processing (NR, MPEG NR, Smooth Gradation, Film Mode) | 4 | Each multi-option |
| Color Space Custom — chromaticity sliders | 12 | 6 colour axes × 2 sliders (Cyan–Red, Magenta–Green). Verified from OSD screenshot |
| Remaining Expert (Clear White, x.v.Color, HDR, Input Lag) | 4 |  |
| Advanced Picture (Auto Calibration) | 4 functions | Pre Check, Adjust, Before/After, Reset |

Note: Counts reflect distinct named controls. Sub-options and slider ranges within each control are additional degrees of freedom.

KEY INSIGHT  The Colour Temperature Gain/Bias section alone (54 adjustments across 9 slots) exceeds the total count of all other basic controls combined — reflecting the depth of calibration available on this projector.

Source: VPL-VW385ES Operating Instructions (Sony Corporation, 47254410M)  ·  Verified 2026