VPL-VW385ES  ·  Complete Menu Inventory

All menu items categorised by type  ·  Green highlight = already implemented in app

Legend

| Badge | Category | Meaning |
| --- | --- | --- |
| ✅ In App | In App | Already implemented and working in the current app |
| 🎛 Adjustable | Adjustable | Can be changed via SDCP SET command — user-facing control |
| ▶ Action | Action | One-shot command (no persistent value) — e.g. Reset, Calibrate |
| 👁 Read-Only | Read-Only | Status/information display only — no SET possible |
| 🔧 Installation | Installation | One-time physical install setting — set once and forget |

Picture

| # | MENU | SUB-SECTION | ITEM NAME | TYPE | OPTIONS / RANGE | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Picture | Core Tone | Calib. Preset | ✅ 🎛 Adjustable | 9 modes: Cinema Film 1/2, Reference, TV, Photo, Game, Bright Cinema, Bright TV, User | Per-input, per-mode storage |
| 2 | Picture | Core Tone | Reset | ▶ Action | — | Resets current Calib. Preset to factory defaults |
| 3 | Picture | Core Tone | Contrast | ✅ 🎛 Adjustable | 0–100 | Label changes to Contrast(HDR) when HDR10/HLG active |
| 4 | Picture | Core Tone | Brightness | ✅ 🎛 Adjustable | 0–100 |  |
| 5 | Picture | Core Tone | Color | 🎛 Adjustable | Slider | Master saturation — all hues equally |
| 6 | Picture | Core Tone | Hue | 🎛 Adjustable | Slider | Master hue rotation (–red / +green) |
| 7 | Picture | Core Tone | Color Temp | ✅ 🎛 Adjustable | D93 / D75 / D65 / D55 / Custom 1–5 |  |
| 8 | Picture | Color Temp | Custom Gain R/G/B | 🎛 Adjustable | 3 sliders per custom slot | 5 custom slots × 3 = 15 Gain sub-controls. Item codes TBD. |
| 9 | Picture | Color Temp | Custom Bias R/G/B | 🎛 Adjustable | 3 sliders per custom slot | 5 custom slots × 3 = 15 Bias sub-controls. Item codes TBD. |
| 10 | Picture | Core Tone | Sharpness | 🎛 Adjustable | Slider | Remote shortcut: SHARPNESS +/– |
| 11 | Picture | Cinema Black Pro | Advanced Iris | ✅ 🎛 Adjustable | Off / Full / Limited | Dynamic iris control |
| 12 | Picture | Cinema Black Pro | Contrast Enhancer | 🎛 Adjustable | Off / Low / Middle / High | Auto bright/dark balance per scene |
| 13 | Picture | Cinema Black Pro | Lamp Control | 🎛 Adjustable | High / Low | Low = deeper blacks, quieter, longer lamp life |
| 14 | Picture | Core | Reality Creation | 🎛 Adjustable | On / Off + sub-controls | Sub: Database (Normal/4K), Resolution slider, Noise Filtering slider |
| 15 | Picture | Core | Motionflow | ✅ 🎛 Adjustable | Off / Smooth High / Smooth Low / Impulse / Combination / True Cinema |  |

Expert Setting

| # | MENU | SUB-SECTION | ITEM NAME | TYPE | OPTIONS / RANGE | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 16 | Expert Setting | Noise | NR (Noise Reduction) | ✅ 🎛 Adjustable | Off / Low / Medium / High / Auto |  |
| 17 | Expert Setting | Noise | MPEG NR | 🎛 Adjustable | Off / Low / Medium / High / Auto | Block noise + mosquito noise from digital compression |
| 18 | Expert Setting | Noise | Smooth Gradation | 🎛 Adjustable | Off / Low / Medium / High | Reduces banding in flat gradient areas |
| 19 | Expert Setting | Noise | Film Mode | 🎛 Adjustable | Auto / Off | Progressive detection for film-sourced content |
| 20 | Expert Setting | Gamma | Gamma Correction | ✅ 🎛 Adjustable | 1.8 / 2.0 / 2.1 / 2.2 / 2.4 / 2.6 / Gamma 7–10 / Off | Gamma 7–10 = custom LDT slots |
| 21 | Expert Setting | Color Correction | Red: Hue | 🎛 Adjustable | −50 to +50 | Item 00h 87h |
| 22 | Expert Setting | Color Correction | Red: Saturation | 🎛 Adjustable | −50 to +50 | Item 00h 88h |
| 23 | Expert Setting | Color Correction | Red: Brightness | 🎛 Adjustable | −30 to +30 | Item 00h 89h |
| 24 | Expert Setting | Color Correction | Yellow: Hue | 🎛 Adjustable | −50 to +50 | Item 00h 8Ah |
| 25 | Expert Setting | Color Correction | Yellow: Saturation | 🎛 Adjustable | −50 to +50 | Item 00h 8Bh |
| 26 | Expert Setting | Color Correction | Yellow: Brightness | 🎛 Adjustable | −30 to +30 | Item 00h 8Ch |
| 27 | Expert Setting | Color Correction | Green: Hue | 🎛 Adjustable | −50 to +50 | Item 00h 8Dh |
| 28 | Expert Setting | Color Correction | Green: Saturation | 🎛 Adjustable | −50 to +50 | Item 00h 8Eh |
| 29 | Expert Setting | Color Correction | Green: Brightness | 🎛 Adjustable | −30 to +30 | Item 00h 8Fh |
| 30 | Expert Setting | Color Correction | Cyan: Hue | 🎛 Adjustable | −50 to +50 | Item 00h 90h |
| 31 | Expert Setting | Color Correction | Cyan: Saturation | 🎛 Adjustable | −50 to +50 | Item 00h 91h |
| 32 | Expert Setting | Color Correction | Cyan: Brightness | 🎛 Adjustable | −30 to +30 | Item 00h 92h |
| 33 | Expert Setting | Color Correction | Blue: Hue | 🎛 Adjustable | −50 to +50 | Item 00h 93h |
| 34 | Expert Setting | Color Correction | Blue: Saturation | 🎛 Adjustable | −50 to +50 | Item 00h 94h |
| 35 | Expert Setting | Color Correction | Blue: Brightness | 🎛 Adjustable | −30 to +30 | Item 00h 95h |
| 36 | Expert Setting | Color Correction | Magenta: Hue | 🎛 Adjustable | −50 to +50 | Item 00h 96h |
| 37 | Expert Setting | Color Correction | Magenta: Saturation | 🎛 Adjustable | −50 to +50 | Item 00h 97h |
| 38 | Expert Setting | Color Correction | Magenta: Brightness | 🎛 Adjustable | −30 to +30 | Item 00h 98h |
| 39 | Expert Setting | Color Space | Color Space | ✅ 🎛 Adjustable | BT.709 / BT.2020 / CS1 / CS2 / CS3 / Custom | Item 00h 3Bh |
| 40 | Expert Setting | Color Space | Custom: Cyan–Red | ✅ 🎛 Adjustable | −100 to +100 | Shown only when Custom selected. Item 00h 76h |
| 41 | Expert Setting | Color Space | Custom: Magenta–Green | ✅ 🎛 Adjustable | −100 to +100 | Shown only when Custom selected. Item 00h 77h |
| 42 | Expert Setting | Other | Clear White | 🎛 Adjustable | Off / Low / High | Emphasises vivid whites. Item code TBD. |
| 43 | Expert Setting | Other | x.v.Color | 🎛 Adjustable | On / Off | Disables gamma correction when On. Item code TBD. |
| 44 | Expert Setting | Other | HDR | ✅ 🎛 Adjustable | Off / HDR10 / HLG / Auto | Item 00h 7Ch |
| 45 | Expert Setting | Other | Input Lag Reduction | 🎛 Adjustable | On / Off | When On: Motionflow/NR/MPEG NR disabled. Item 00h 99h |

Advanced Picture

| # | MENU | SUB-SECTION | ITEM NAME | TYPE | OPTIONS / RANGE | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 46 | Advanced Picture | Auto Calibration | Pre Check | ▶ Action | — | Measures panel color vs factory reference; reports dE. Takes minutes. |
| 47 | Advanced Picture | Auto Calibration | Adjust | ▶ Action | — | Performs automatic panel color calibration. Lens moves. Takes minutes. |
| 48 | Advanced Picture | Auto Calibration | Before/After | ▶ Action | — | Toggles factory vs calibrated at fixed frequency for visual comparison |
| 49 | Advanced Picture | Auto Calibration | Reset | ▶ Action | — | Reverts panel calibration to factory defaults |

Screen

| # | MENU | SUB-SECTION | ITEM NAME | TYPE | OPTIONS / RANGE | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 50 | Screen |  | Aspect | 🎛 Adjustable | Normal / 1.85:1 Zoom / 2.35:1 Zoom / V Stretch / Squeeze / Stretch | Item code TBD. |
| 51 | Screen |  | Blanking: Left | 🎛 Adjustable | Slider | Crop left edge. Item code TBD. |
| 52 | Screen |  | Blanking: Right | 🎛 Adjustable | Slider | Crop right edge. Item code TBD. |
| 53 | Screen |  | Blanking: Top | 🎛 Adjustable | Slider | Crop top edge. Item code TBD. |
| 54 | Screen |  | Blanking: Bottom | 🎛 Adjustable | Slider | Crop bottom edge. Item code TBD. |
| 55 | Screen |  | Picture Position | 🔧 Installation | Save/Delete/Select (up to 5 presets) | Stores lens position + aspect + blanking. Triggers lens motor movement. |

Function

| # | MENU | SUB-SECTION | ITEM NAME | TYPE | OPTIONS / RANGE | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 56 | Function | 3D Settings | 2D-3D Display | 🎛 Adjustable | Auto / 3D / 2D | Item code TBD. |
| 57 | Function | 3D Settings | 3D Format | 🎛 Adjustable | Simulated 3D / Side-by-Side / Over-Under | Item code TBD. |
| 58 | Function | 3D Settings | 3D Brightness | 🎛 Adjustable | High / Standard | Item code TBD. |
| 59 | Function | 3D Settings | 3D Depth Adjust | 🎛 Adjustable | −2 / −1 / 0 / +1 / +2 | Item code TBD. |
| 60 | Function | 3D Settings | Simulated 3D Effect | 🎛 Adjustable | High / Middle / Low | Item code TBD. |
| 61 | Function |  | Dynamic Range | 🎛 Adjustable | Auto / Limited / Full | Sets HDMI input level. Item code TBD. |
| 62 | Function |  | HDMI Signal Format | 🎛 Adjustable | Standard / Enhanced | 4K HDMI format. Item code TBD. |
| 63 | Function |  | Test Pattern | 🎛 Adjustable | On / Off | Green test pattern for lens focus/zoom/shift. Item code TBD. |
| 64 | Function |  | Settings Lock | 🎛 Adjustable | Off / Level A / Level B | Locks menu items to prevent accidental changes. Item code TBD. |

Setup

| # | MENU | SUB-SECTION | ITEM NAME | TYPE | OPTIONS / RANGE | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 65 | Setup |  | OSD Status | 🔧 Installation | On / Off | Turns on-screen display on/off. Item code TBD. |
| 66 | Setup |  | Language | 🔧 Installation | Multiple | Menu display language. Item code TBD. |
| 67 | Setup |  | Menu Position | 🔧 Installation | Bottom Left / Center | Item code TBD. |
| 68 | Setup |  | High Altitude Mode | 🔧 Installation | On / Off | Required above 1,500 m. Item code TBD. |
| 69 | Setup |  | Remote Start | 🔧 Installation | On / Off | Network wake-on-LAN. Item code TBD. |
| 70 | Setup |  | Network Management | 🔧 Installation | On / Off | Keep network active in standby. Item code TBD. |
| 71 | Setup |  | Power Saving | 🔧 Installation | Off / Standby | Auto standby after 10 min no signal. Item code TBD. |
| 72 | Setup |  | All Reset | ▶ Action | — | Factory reset all settings (not Lamp Timer). Item code TBD. |

Installation

| # | MENU | SUB-SECTION | ITEM NAME | TYPE | OPTIONS / RANGE | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 73 | Installation |  | Image Flip | 🔧 Installation | Off / HV / H / V | Ceiling/rear projection orientation. Item code TBD. |
| 74 | Installation |  | Lens Control | 🔧 Installation | On / Off | Locks lens motor buttons. Item code TBD. |
| 75 | Installation |  | Anamorphic Lens | 🔧 Installation | 1.24x / 1.32x | Anamorphic conversion ratio. Item code TBD. |
| 76 | Installation |  | Trigger Select | 🔧 Installation | Off / Power / V Stretch / 2.35:1 Zoom | 12V trigger output. Item code TBD. |
| 77 | Installation |  | IR Receiver | 🔧 Installation | Front & Rear / Front / Rear | Remote control detector location. Item code TBD. |
| 78 | Installation |  | Panel Alignment | 🔧 Installation | Shift/Zone per R/B vs G | Color convergence correction per zone. Complex procedure. Item code TBD. |
| 79 | Installation |  | Network Setting | 🔧 Installation | DHCP / Manual IP | IP address configuration. Managed via projector OSD or web UI. |

Information

| # | MENU | SUB-SECTION | ITEM NAME | TYPE | OPTIONS / RANGE | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 80 | Information |  | Model Name | 👁 Read-Only | — |  |
| 81 | Information |  | Serial Number | 👁 Read-Only | — |  |
| 82 | Information |  | Signal Type | 👁 Read-Only | — | Resolution + 2D/3D + frame format |
| 83 | Information |  | Color Format | 👁 Read-Only | — | YCbCr / RGB |
| 84 | Information |  | Color Space | 👁 Read-Only | — | BT.601 / BT.709 / BT.2020 |
| 85 | Information |  | HDR Format | 👁 Read-Only | — | SDR / HDR(HDR10) / HDR(HLG) |
| 86 | Information |  | Software Version | 👁 Read-Only | — |  |
| 87 | Information |  | Lamp Timer | 👁 Read-Only | — | Total hours of lamp usage |

Power/Input

| # | MENU | SUB-SECTION | ITEM NAME | TYPE | OPTIONS / RANGE | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 88 | Power/Input |  | Power On | ▶ Action | — | SDCP command. Item 01h 30h value 01h |
| 89 | Power/Input |  | Power Off | ▶ Action | — | SDCP command. Item 01h 30h value 00h |
| 90 | Power/Input |  | Input Select | 🎛 Adjustable | HDMI 1 / HDMI 2 | Item code TBD. |

Source: VPL-VW385ES Operating Instructions (Sony, 47254410M)  ·  Generated 2026-04-16  ·  90 items total